Public ip  =>   unique address (work globally on the internet)

Private ip => repeated address (only in lan )

Private ip ranges :

Class A   10.0.0.0   to 10.255.255.255

Class B   172.16.0.0   to  172.31.255.255

Class C  192.168.0.0   to 192.168.255.255

Network address translation

Static Nat => 1 to 1 mapping (public to one private )

Dynamic Nat => (many to many )

PAT    => Port address translation  (many to one )

Ip addressing method

Manual,  (static)

Automated   (DHCP  , APIPA)

DHCP  => DORA Function   (Discover offer request acknowledgement)

D -> Any DHCP Server available (Router)

O -> DHCP server replies with an IP offer

R ->  Client sends request to accept the offered IP

A -> Acknowledgement (Server confirms and assign the ip officially)

APIPA  => Automated Private IP Addressing

DHCP is not available => automatically assigns itself an ip address

TCP/IP

Uses a three-way handshake to link sender and receiver

Ping

Tracert

Ipconfig, ipconfig /all

nslookup