Bounded network media => guided media (wired)

Twisted pair cable (Ethernet)  => LAN

Copper wires (twisted cables)

Coaxial cable  => copper core surrounded by insulation and metal shield

It carries datas like cable tv and broadband internet

And is outside noise

Fiber optic cable  =>  uses hair-thin strands  , light pulses and high-speed

Block outside noise completely ,

Unbounded Network media (unguided media ) (wireless)

Data travels through air or space  , no wires,

Radio waves (wifi, bluetooth)

Penetrate walls, all direction from the source

Microwaves (satellite, cell towers)

Data travels in straight line

Infrared  => very short -range  and    strict line of sight

Don’t penetrate

Noise control

Electromagnetic interference  or noise

How we can control

Twisting the wires

Shielding

using light instead -> electrical noise immune

HUB  ->

Switch ->

Signalling methods =>

Baseband => electrical signal (0s, 1s)   => single path (integrity)

broadband => analog signal

OSI model  => Open – system interconnection

Layer 7 : Application Layer    => user interaction

Layer 6 : Presentation Layer   => translates,   encryption , compress

Layer 5 : Session Layer   =>  creates session between devices

Layer 4: Transport layer =>  divides large file into small packets  -> each packet has sequence number , source ip and destination ip

Layer 3: Network layer  => Router (delivery over the network )

Layer 2 :Data link Layer  => switch ( mac address)

Layer 1 : Physical layer   => transmits raw bit over physical cables

NAT   Steps

ARP -> Address Resolution Protocol   (IP to MAC)

RARP  -> Reverse Address Resolution Protocol (MAC to IP)

NETBIOS  -> Network basic input output system (instead of IP, Based on system name ) Supports only windows

Wins -> Windows Internet name service  (support non-windows family also support  )

TCP model

Application layer

Transport Layer  =>  (TCP, UDP)

Transmission control protocol ->  Ensure delivery guarantee , slower compare to udp

User datagram protocol	 -> doesn’t ensure delivery guarantee , faster because if any    data loss in transmission it doesn’t take care

Internet layer   -> Router

Network access (Link layer) ->

1gb => 1024 MB

1mb => 1024 kbs

1kb => 1024 bytes

1 bytes => 8 bits

1 bit => 0 or 1

IP types

Ipv4 ->  32 bits   =>   address count 4.3 billion  ->   Decimal

Ipv6 -> 128 bits  => unlimited    ->  Hexadecimal

Number system

Binary

Decimal

HexaDecimal

Structure of IPv4   -> 192.168.1.1

4 octet  ->   32 bits

1 octet -> 8 bits

Ranges (0 – 255)

Never start with 0

Base value

Binary – 0,1  => 2

Decimal -> 0 to 9   => 10

Hexadecimal -> 0 to 15 => 16

Ipv4 -> 192.168.1.0   => binary

| 2^7 | 2^6 | 2^5 | 2^4 | 2^3 | 2^2 | 2^1 | 2^0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |

Left to right = 128 + 64 => 192

1100 0000. 1010 1000. 0000 0001. 0000 0000

172.16.9.0

254.255.230.40

Ipv6 => 8 octet

1 octet => 16 bits

8*16 => 128 bits

Ipv6 => 3F:5B:BB3  => 315:511:111113

IP classes

Class A  = 0 – 127  => 1-126  (Loop address)

Class B = 128 – 191

Class c = 192 – 233

Class D = 234 – 239  => Multicast (many to many)

Class E  = 240 – 255 => Research & Development

MAC => Media access control (physical address)

NIC -> Network interface card

Mac -> 4 octet

3 octet -> same based on manufacturer  (OUI) (organsically unique identifier)

1 octet -> unique address

Subnetting ->  Process of dividing larger to smaller network

Types of subnetting

Classful subnetting  - FLSM   (Full length Subnet mask)

Classless subnetting – VLSM (Variable length subnet mask) , CIDR

| IP class | Subnet | Network Formula | Host formula |
| --- | --- | --- | --- |
| Class A | N.H.H.H | N = (2^N) -2 | H = (2^H) -2 |
| Class B | N.N.H.H | N = (2^N) | H = (2^H) -2 |
| Class C | N.N.N.H | N = (2^N) | H = (2^H) -2 |
| Class D | N.N.N.N | - | - |
| Class E | N.N.N.N | - | - |

Ipv4 => 32 bits => 2^32  => 4.3 billion

1000 0000 0000 0000 0000 0000 0000 0000  =>  128.0.0.0

1100 => 192.0.0.0

1110 => 224.0.0.0

0000 0000 => 0

1111 1111 => 255

Subnet mask => System able to find out the which is network part and host part

Classful -> FLSM

192.168.1.0    =>  Class C

Subnet mask = 255.255.255.0

Inverse mask = Network range – subnet mask

= 255.255.255.255  - 255.255.255.0

= 0.0.0.255

Ranges   = Inverse mask + network ID

= 0.0.0.255 + 192.168.1.0   => 192.168.1.255

192.168.1.0   - 192.168.1.255

192.168.1.1  - 192.168.1.254   =>     (2^H) -2   =>  (2^8) -2  => 256 -2 => 254

172.16.0.0

172.16.0.0    - 172.16.255.255

172.16.0.1  - 172.16.255.254

32 =>

16 => 10  01  11   00  => 4

192.168.1.0 -> Class c => Subnet mask  =>  N.N.N.H

VLSM   => Variable length subnet mask

172.18.0.0   - required host – 100

| 2^7 | 2^6 | 2^5 | 2^4 | 2^3 | 2^2 | 2^1 | 2^0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |

Host = (2^H) -2

= (2^7)-2  => 126

Network bits – no of host bits

= 32 – 7  => 25 //

Subnet mask =>  8.8.8.1  => 255.255.255.128

Subnet = 2^S  => 2^1 => 2

Inverse mask = 255.255.255.255  - 255.255.255.128

= 0.0.0.127

Ranges = id + mask

= 172.168.0.0   + 0.0.0.127

= 172.168.0.127

Network id = 172.168.0.0

Broadcast address = 172.168.0.127

Usable ip  =  172.168.0.1  to 172.168.0.126

172.168.0.0  => required host 1000

22 => 8.8.6.0  => 255.255.252.0

CIDR => classes Inter-domain Routing  // slash value

192.168.0.0  / 16

= 8.8.0.0  => 255.255.0.0

Inverse => 0.0.255.255

Range = 192.168. 255.255

Usable => 192.168.0.1  to 192.168.255.254

192.168.1.0 / 27

- 8.8.8.3     => 255.255.255.224

- Subnet = 2^S   ->  2^3  => 8

- 8.8.8.3  => find subnet and host  => 8 , 30

Subnet problem

Required subnet

- Separate the host bits and write

- Write  host bit place value from left to right

- Based on requirement move a line

10.0.0.0/8     => required subnet = 4

| 128 | 64 | 32 | 16 | 8 | 4 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

10.

10.         2           4              8           16              32         64         128         256

Public ip and private ip

Private ip ranges  => IANA  (internet assigned numbers authority)

Class A   -  1 to 126  => 10.0.0.0   to 10.255.255.255

Class B   =>  172.16.0.0   to 172.31.255 255

Class C  => 192.168.0.0  to 192.168.255.255

Ip assigning method   => manual, automated

Static =>

Dynamic => DHCP    dynamic host configuration protocol  , APIPA

Automated private ip addressing

DORA function

Discover  => Any DHCP server available (Router)

Offer  => DHCP server replies with an ip offer

Request  => Client sends request to accept the offered IP

Acknowledgement  => Server confirms and assign the ip officially

Apipa is a feature in networking where  a device automatically assigns itself an ip address

When dhcp server is not available